- Connect your laptop and a friend's phone to the same Wi-Fi network, then use the ping command to check if both devices can communicate with each other.<br><br><em><strong>Hint:</strong> On Windows, use 'ping [IP address]' in Command Prompt; on Android, try a ping app or use Termux.</em>

Ans :-  To check whether a laptop and a friend's phone can communicate on the same Wi-Fi network.

Steps:

- Connect the laptop and the friend's phone to the same Wi-Fi network.

- On the laptop, open Command Prompt by pressing Win + R, typing cmd, and pressing Enter.

- Type ipconfig and note the laptop's IPv4 address.

- On the phone, open Wi-Fi settings and check its IP address.

- On the laptop, type the following command using the phone's IP address:

ping 192.168.1.5

Example IP address only; replace it with the phone's actual IP.

- If you receive Reply from..., the devices can communicate.

- If you receive Request timed out, the phone may be blocking ping requests, the Wi-Fi may isolate devices, or the phone may be disconnected.

- Run the traceroute (tracert on Windows or traceroute on Mac/Linux) command to www.youtube.com from your device and note down the number of hops it takes to reach the destination.

Ans :-  To find the network hops between your device and YouTube.

Steps on Windows:

- Open Command Prompt.

- Type the following command:

tracert www.youtube.com

- Press Enter and wait for the results.

- Count the numbered hop lines to estimate the number of hops.

Tracing route to www.youtube.com

1     2 ms     1 ms     2 ms  192.168.1.1

2    10 ms    12 ms    11 ms  ISP Gateway

3    18 ms    20 ms    19 ms  Network Router

4     *        *        *     Request timed out.

The traceroute command shows the route taken by network packets toward YouTube. The actual number of hops depends on the network. A * or timeout does not necessarily mean the route has failed.

- Disconnect one device from the Wi-Fi, then try pinging it again from your laptop and describe the error message you receive.

Ans :-

Steps:

- First, connect both devices to the same Wi-Fi.

- Ping the phone from the laptop and check the result.

- Disconnect the phone from Wi-Fi.

- Run the same ping command again.

Expected error message

Request timed out.

Another possible message is

Destination host unreachable.

- After disconnecting the phone, the laptop usually cannot communicate with it over the Wi-Fi network. The exact error depends on the network and operating system.

4.  Create a simple table listing the IP addresses of all devices connected to your home Wi-Fi (use 'arp -a' on Windows), and identify which device belongs to which person or gadget.

Ans :-  On Windows, open Command Prompt and type:

arp -a

This displays IP addresses and associated MAC addresses known to your computer. It may not show every connected device, so check your router's connected-device list for a more complete list.

| IP address | Device | Belongs to |
| --- | --- | --- |
| 192.168.1.1 | Wi-Fi router | Family |
| 192.168.1.2 | Laptop | Me |
| 192.168.1.5 | Smartphone | Friend |
| 192.168.1.6 | Smart TV | Family |
| 192.168.1.7 | Printer | Family |

5. Write a one-page report explaining step-by-step how you set up the network, tested connectivity using ping and traceroute, and what results or issues you observed.<br><br><em><strong>Hint:</strong> Include screenshots or command outputs if possible.</em>

Ans :-

Objective: To connect two devices to the same Wi-Fi network and test network connectivity using ping and traceroute commands.

Procedure:

First, I connected my laptop and my friend's smartphone to the same Wi-Fi network. I checked the IP addresses of both devices. On the Windows laptop, I opened Command Prompt and used the ipconfig command to find the laptop's IPv4 address.

Next, I used the ping command followed by the smartphone's IP address to test communication between the devices. If replies were received, it indicated that the devices could communicate. If the request timed out, I checked the Wi-Fi connection and considered whether the phone or router was blocking ping requests.

After that, I used the tracert www.youtube.com command to trace the network route to YouTube. The command displayed the hops between my device and the destination. I counted the numbered hop lines and observed the response times. Some hops could show asterisks if routers did not respond.

I then disconnected the smartphone from Wi-Fi and repeated the ping test. The connection usually failed, and a message such as “Request timed out” could appear.

Finally, I ran the arp -a command to view IP addresses and MAC addresses known to my laptop. I used these details, along with the router's connected-device list, to identify the devices on the network.